MicroBoard Studio

Interactive Microcontroller Board, Pin Visualizer & Smart Upload Dashboard

Project concept, innovations, and technical build plan

# 1. Project Overview

MicroBoard Studio is an Arduino-compatible custom development board paired with a software platform that makes microcontroller pins, peripherals and uploads easy to understand and use.

The core idea is not "another Arduino clone" but a learning and configuration ecosystem:

Learn → Configure → Simulate → Generate Code → Connect to Real Board

### Core software features

- Pin Explorer – click any pin to see its Arduino name, MCU port/bit (e.g. D13 → PB5), and capabilities (PWM, ADC, SPI, etc.).

- Peripheral Highlighting – select UART / SPI / I²C and the board highlights the relevant pins.

- Register View – shows how pinMode() and digitalWrite() map to DDRx / PORTx / PINx bits.

- Code → Hardware Visualizer – trace a line of code down to the physical pin.

- Pin Conflict Detection – warns when one pin is assigned to two functions.

- Learn Mode – quizzes on pins, registers and peripherals.

# 2. Innovations That Win Judges

Judges will ask: "How is this different from Wokwi, Tinkercad or Fritzing?" Those tools simulate a fake board or draw diagrams. The features below give a strong answer.

## 2.1 Live link to the real board (most important)

Simulators show a fake board. MicroBoard shows the real one.

- The board runs a small monitor firmware that streams real DDRx, PORTx, PINx register values and ADC readings over USB.

- The UI updates live: press a button on the physical board and D2 lights up on screen while the PIND bit flips.

- Works both ways: clicking D13 in the UI toggles the real LED.

This single feature justifies building both the hardware and the software.

## 2.2 Pins that protect and report themselves

This is what makes the custom PCB worth building instead of a clone.

- Each header pin has current sensing and protection; the board detects shorts or overcurrent and the UI shows "D7 is shorted to GND".

- Optional: a small RGB LED next to each pin showing its mode (input, output, PWM, I²C…).

- Catches wrong wiring before it damages the chip – a very common beginner problem.

## 2.3 Checker for hidden pin conflicts

Real Arduino gotchas most students do not know. The tool reads the sketch and warns:

- Servo library used → PWM on D9 and D10 stops working (Timer1 is taken).

- tone() used → PWM on D3 and D11 breaks (uses Timer2).

- Serial running while D0/D1 are used as GPIO → conflict.

- analogWrite() on a non-PWM pin → it switches fully on/off instead of fading.

- I²C running while A4/A5 are read as analog inputs → conflict.

Each warning is highlighted on the board graphic.

## 2.4 Watch the actual signals on each pin

- Shows PWM waveform and duty cycle.

- Decodes UART bytes travelling over TX/RX.

- Decodes I²C traffic, e.g. "Address 0x3C, write 0x00 0xAF".

## 2.5 Automatic pin planner

User picks parts (e.g. OLED + 2 servos + DHT11 + 3 LEDs). The tool:

- Assigns pins while avoiding the conflicts from 2.3.

- Draws the wiring diagram.

- Generates starter code.

- Verifies the result against the real board using the live link (2.1).

## 2.6 How to pitch it

- Pick 3 main features, not 10. Recommended: 2.1, 2.2 and 2.3. Add 2.4/2.5 if time allows.

- 2-minute demo: write code → checker flags a Servo/PWM conflict → fix it → upload → live UI mirrors the board → short a pin on purpose → board catches it.

- Tagline: "Simulators show a fake board. MicroBoard shows you what your real board is doing, and stops you from breaking it."

- Avoid "we added AI" as the headline. An AI tutor can be a small extra only.

# 3. Smart Upload Dashboard (Arduino IDE Replacement)

The Arduino IDE frequently shows port errors, makes board selection confusing, hides Upload under menus, and tucks the Serial Monitor/Plotter into corners. The dashboard fixes this and becomes the front door to MicroBoard Studio.

## 3.1 What Arduino IDE 2 already does

Judges may have used Arduino IDE 2, which already has a toolbar Upload button (Ctrl+U), some automatic board detection, and a serial plotter. So "a cleaner Arduino IDE" is not enough. The project wins by fixing what it still gets wrong:

| Problem | What Arduino IDE does | What MicroBoard does |
| --- | --- | --- |
| Port errors | Shows raw errors like programmer is not responding | Explains the actual cause and the fix in plain words |
| Clone boards (CH340) | Shows "Unknown board" | Probes the board to identify the chip |
| Wrong board for the code | Compiles anyway, fails later | Warns: "Sketch uses esp_camera.h but an Uno is connected" |
| Serial Monitor holding the port | Upload fails – port busy | Closes monitor before upload, reopens after |
| Monitor and plotter | Separate panels, hard to find | Always visible next to the code |

Selling point: the Port Doctor plus smart board detection. A nicer layout is only a bonus.

## 3.2 Overall architecture

┌──────────────────────────────────────────────────────┐

│                MICROBOARD DASHBOARD (UI)             │

│  ┌───────────┐  ┌──────────────┐  ┌───────────────┐  │

│  │ Device    │  │ Code editor  │  │ Serial monitor│  │

│  │ panel     │  │ + Upload     │  │ + Plotter     │  │

│  └───────────┘  └──────────────┘  └───────────────┘  │

└──────────────────────────┬───────────────────────────┘

│

┌──────────────────────────┴───────────────────────────┐

│                  BACKEND SERVICES                    │

│  Device Watcher → Board Identifier → Port Doctor     │

│  Sketch Analyzer → Build/Upload Manager              │

│  Serial Manager (one owner of the port)              │

└──────────────────────────┬───────────────────────────┘

│

arduino-cli   esptool   avrdude   serialport

│

USB / Board

Important: do not build a compiler or uploader. Use arduino-cli, the official command-line tool that Arduino IDE 2 itself runs on. It handles compiling, uploading, board cores and libraries. The project builds the smart layer on top.

## 3.3 Device Watcher – spotting USB plug and unplug

- List serial ports every second, or listen for USB events.

- When a new port appears, read its VID/PID (USB ID that identifies the chip):

| VID:PID | Chip | Usually found on |
| --- | --- | --- |
| 2341:0043 | Genuine Arduino | Uno R3 |
| 1A86:7523 | CH340 | Clone Uno, Nano, some ESP boards |
| 10C4:EA60 | CP2102 | ESP32 DevKit, NodeMCU |
| 0403:6001 | FTDI | Older Nano, FTDI adapters |
| 303A:1001 | ESP native USB | ESP32-S3, ESP32-C3 |
| 2E8A:000A | RP2040 | Raspberry Pi Pico |

arduino-cli board list --format json provides most of this for free.

## 3.4 Board Identifier – the key innovation

A CH340 chip could be on an Uno clone, a Nano clone or an ESP32, so VID/PID alone is not enough. The dashboard probes the board:

New port found

│

├─ Genuine VID/PID? → done (e.g. "Arduino Uno")

│

└─ Generic USB chip (CH340/CP2102/FTDI)?

│

├─ Try esptool chip_id → replies? → "ESP32 / ESP32-S3 / ESP8266"

│

└─ Try AVR bootloader at 115200 baud → replies? → ATmega328P (Uno)

Try AVR bootloader at 57600 baud  → replies? → ATmega328P (old Nano)

- esptool reads the chip type directly from any ESP board.

- avrdude reads the chip signature: 1E 95 0F = ATmega328P.

- Limitation: Uno and Nano use the same chip, so sometimes they cannot be told apart. Ask the user once and remember the answer for that board.

## 3.5 Sketch Analyzer – which board is the code for?

| Found in code | Means |
| --- | --- |
| esp_camera.h | ESP32-CAM |
| WiFi.h, WebServer.h | ESP32 / ESP8266 |
| ESP8266WiFi.h | ESP8266 |
| analogRead(A6) | Nano (the Uno has no A6) |
| Pin numbers above 19 | Not an Uno or Nano |

The result is compared with the connected board: "⚠️ Your code is for ESP32-CAM, but an Arduino Uno is plugged into COM6." This catches the mistake before a long compile that would fail anyway.

## 3.6 Port Doctor – turning errors into fixes

| What is really wrong | What the user normally sees | What the dashboard says |
| --- | --- | --- |
| Charge-only USB cable | No port at all | "Board powered but no data. Try a different USB cable." |
| CH340 driver missing (Windows) | Unknown device | "Install the CH340 driver" (with link) |
| Port held by another app | Access is denied / port busy | "COM6 is in use by another program. Close the Serial Monitor or Cura." |
| Wrong board selected | programmer is not responding | "This looks like an older Nano. Switch to the old bootloader." |
| ESP32 not in boot mode | Wrong boot mode detected | "Hold the BOOT button while uploading." |
| ESP32-CAM | Upload fails | "ESP32-CAM has no USB chip. Use an FTDI adapter and connect IO0 to GND." |
| Linux permissions | Permission denied /dev/ttyUSB0 | "Run: sudo usermod -aG dialout $USER" |

A table of error patterns mapped to explanations is enough to implement this, and it makes a great demo.

## 3.7 Serial Manager – one owner for the port

Many port errors happen because the monitor and uploader fight over the same port. One service owns the port at all times:

- On Upload, close the monitor.

- Run the upload.

- Reopen the monitor at the same baud rate.

Extras:

- Baud detection: if output looks like garbage (⸮⸮⸮), try other common baud rates and pick the one that yields readable text.

- Plotter: parse lines like temp:24.5,hum:60 into separately labelled graph lines automatically.

The same serial connection later powers the live pin view (2.1), so it is not throwaway work.

## 3.8 Recommended tech stack

| Layer | Choice | Why |
| --- | --- | --- |
| Desktop app shell | Electron | Needs full USB/serial access a website cannot get. JavaScript-based, easiest for a student team. |
| UI | React + TypeScript + Tailwind | Fast to build, good component libraries |
| Code editor | Monaco Editor | The editor inside VS Code – highlighting and autocomplete |
| Compile & upload | arduino-cli (bundled) | Official tool, supports every board |
| Serial ports | serialport (npm) | Mature, provides VID/PID |
| ESP detection | esptool | Reads the ESP chip type |
| AVR detection | avrdude (ships with arduino-cli) | Reads the ATmega signature |
| Plotter | uPlot or Chart.js | uPlot handles fast live data smoothly |
| Saved settings | JSON file or SQLite | Remembers which board is on which port |

Alternatives:

- Tauri instead of Electron – much smaller app, but the backend is in Rust (harder).

- Python-only team: PyQt + pyserial + arduino-cli – same design, different tools.

## 3.9 Build plan

| Week | Milestone |
| --- | --- |
| 1 | Electron app running; list ports with serialport and update live on plug/unplug |
| 2 | Connect arduino-cli for compile & upload – one Upload button with progress |
| 3 | Serial monitor and plotter, with port close/reopen around uploads |
| 4 | Board Identifier with esptool and avrdude probing |
| 5 | Sketch Analyzer and code/board mismatch warning |
| 6 | Port Doctor error table and polish |

## 3.10 Why judges will like it

- They have hit the problem themselves. Almost everyone who has used Arduino has seen a port error.

- It is technically serious. Probing bootloaders, reading chip signatures and translating errors is real embedded work, not just a nicer UI.

- It connects to the rest of the project. The dashboard detects the board; the pin explorer and live board view then use that same connection.

- It is easy to demo: plug in a clone board → identified → open an ESP-CAM sketch → mismatch warning → plug in the right board → one-click upload → plotter starts.